JS Air Curtains Kool

Air Curtain Specification Summary

- The Air Curtain shall have a self-supporting casing made of galvanized plated steel, finished in structural epoxy-polyester RAL 9016 as standard. Brushed stainless steel construction shall be available on request.

- The Air Curtain shall use low noise centrifugal double inlet fans driven by an external rotor motor with built in thermal protection.

- The Air Curtain shall be available as standard in a choice of 2 different fan powers to suit different mounting heights and environments using AC type fans.

- The Air Curtain shall also be available with an option of EC fans for energy saving. There will be a choice of 2 different fan powers to suit different mounting heights for this option.

- The fans shall be capable of 5 speed selections.

- The Air Curtain shall be specifically designed for use on cold store and freezer doors.

- The Air Curtain shall also be ideal for air separation of internal doors and other specialist applications.

- The Air Curtain shall be available in a choice of sizes nominally from 1.0m to 3.0m in 0.5m increments.

- The Air Curtain shall measure 260mm high and 370mm deep including intake grille.

- The unit shall be suitable for exposed mounting above a door using M8 threaded drop rods.

- An optional base plate shall be available to allow vertical mounting to side of the doorway.

- The inlet grille shall be perforated with filter function. For servicing, in normal environments, the grille only has to be periodically wiped or vacuumed.

- The Air Curtain shall be available as ambient unheated only.

- The Air Curtain will use anodised aluminium blow-out vanes, airfoil shaped, adjustable from 0 to 15º each side of vertical for airflow direction control.

- The Air Curtain shall include as standard a manual controller, 7m of telephonic cable with RJ45 (Plug & Play) connectors and a hand held infra-red remote.

- The manual controller shall be capable of BMS/PLC/Timer etc. enable and disable by a suitable volt free contact.

- An optional controller shall be available which will provide run on when the unit is used with door switches.

- The Air Curtain shall be available with optional IP55 rated components for use in damp and humid environments such as wash down areas etc.

- The system is to be supplied by:

JS Air Curtains

Member of the Condair Group

Artex Avenue

Rustington

LITTLEHAMPTON

West Sussex

BN16 3LN

Tel: +44 (0) 1903 858 656 			Fax: +44 (0) 1903 850345

e-mail: sales@jsaircurtains.com		website: www.jsaircurtains.com